|  |  |  | ANJANI PORTLAND CEMENT LIMTIED |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Ref No. |  | OPER/SOP-2 |  | Dept: | Operations |  |  |
|  |  |  | Supersedes |  | Nil |  | Page : | 1 of 4 |  |  |
| Version No./Dt. |  | 01/01.03.2018 |  | Rev.No./Dt |  | 00/01.03.2018 | Next Review Dt. | 28.02.2021 | Effective Dt. | 01.03.2018 |
| Title | Standard Operating Procedure for Kiln start up and operation. |  |  |  |  |  |  |  |  |  |

1.0 Purpose:

To en sure standard operation of   Kiln start up and safe operation.

2.0 Scope:

Pre-heater,  Kiln, cooler, ESP and RABH.

3.0 Responsibility:

H.O.D (Operation)/Section Incharge (Operation) is responsible for implementation of this SOP.

4.0 Pre requisites

- Before light up of the kiln get clearance for Kiln, Raw mills, coal mill & ESP from Service departments by phone.

- Do fire-ball drop test to ensure feed pipes and cyclone cones are free from blockage

- Ensure all groups are in healthy condition by taking group trails up to maximum extent.

- Ensure the diesel firing nozzle is clean / fire wood is sufficient

- Check the Diesel firing system for its readiness before light up.

- All pre-heater cyclones and feed pipes to be thoroughly checked and all pre-heater doors to be closed.

- Ensure all cyclones feed pipe flaps or operating properly if required flaps are kept in open condition.

5.0 Procedure

- Start RABH fan with minimum speed and 100% damper open, and start Primary air fan, keep desired primary air pressure.

- Ensure Kiln hood draught is in the range of -2 to -3 mmWC by adjusting RABH fan speed and PH fan damper.

- Start Diesel oil pumps with recirculation to increase oil temp.

- If oil temp reach 900C light up the kiln with diesel firing system and gradually rise the kiln inlet temperature by increasing the draught and diesel quantity.

- Inform to mechanical dept for pouring of oil on supporting roller bearings manually.

- Put Kiln auxiliary drive in auto barring with an interval of 30 minutes.

- When the kiln inlet temperature reaches about 4500C start the kiln coal firing of about 0.5 to 1.0 MT per hour.

- Change Kiln barring interval to 15 minutes.

- Raise the Kiln inlet temp 50-80 0C/hr by increasing the kiln coal and diesel quantity.

- Ensure kiln inlet “CO” is below 0.2%.If necessity increase PH draught by opening PH fan damper and RABH fan speed.

|  |  |  | ANJANI PORTLAND CEMENT LIMTIED |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Ref No. |  | OPER/SOP-2 |  | Dept: | Operations |  |  |
|  |  |  | Supersedes |  | Nil |  | Page : | 2 of 4 |  |  |
| Version No./Dt. |  | 01/01.03.2018 |  | Rev.No./Dt |  | 00/01.03.2018 | Next Review Dt. | 28.02.2021 | Effective Dt. | 01.03.2018 |
| Title | Standard Operating Procedure for Kiln start up and operation. |  |  |  |  |  |  |  |  |  |

- When the kiln inlet temperature reaches about 4500C start the kiln coal firing of about 0.5 to 1.0 MT per hour.

- Change Kiln barring interval to 15 minutes.

- Raise the Kiln inlet temp 50-80 0C/hr by increasing the kiln coal and diesel quantity.

- Ensure kiln inlet “CO” is below 0.2%.If necessity increase PH draught by opening PH fan damper and RABH fan speed.

- Once the kiln inlet temperature reaches to ~ 8000C keep the kiln in continuous rotation with auxiliary drive.

- Make All Kiln auxiliaries such as Clinker transport, Dust transport, Cooler fans, and cooler drives are in operation.

- Kiln feed system to be operated in set point of 20 t/h

- Maintain PH-fan outlet pressure: -10 to –20 mmWC always by adjusting RABH fan speed.

- Switch on ESP high-tension system

- Maintain firing hood pressure:  – 2 to -5 mmWC by varying cooler ESP fan speed.

- Ensure that the RABH inlet temperature is not more than 220˚C. Make sure temp control with fresh air damper is in auto mode.

- Gas analyzers and NOx and SOx levels are in operating condition.

- When the Kiln Inlet temperature reaches 1050˚C the following steps to be taken rapidly in the sequence.

- Stop the Kiln auxiliary and start Kiln main drive at 0.5rpm.

- Start the Pre-heater ID-fan  & increase pre-heater drafts by opening Pre-heater fan rpm

- Increase the speed of the Kiln to 1 rpm.

- Start the Calciner with minimum fuel firing

- Than Kiln feed to be taken.

- Start the cooler drives and adjust cooler fans speed & ESP fan.

- Increase gradually Pre-heater fan speed, Kiln speed & feed, fuel to the Kiln and PC to optimize the kiln operation.

- Increase the Primary air fan pressure  max by fan rpm

- Increase Cooler fans flow set point and cooler drive strokes set point, depending on the clinker bed thickness in the Cooler and proportional to the Kiln feed.

- After stabilizing the kiln feed at, put all process control loops in auto mode for smooth operation.

|  |  |  | ANJANI PORTLAND CEMENT LIMTIED |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Ref No. |  | OPER/SOP-2 |  | Dept: | Operations |  |  |
|  |  |  | Supersedes |  | Nil |  | Page : | 3 of 4 |  |  |
| Version No./Dt. |  | 01/01.03.2018 |  | Rev.No./Dt |  | 00/01.03.2018 | Next Review Dt. | 28.02.2021 | Effective Dt. | 01.03.2018 |
| Title | Standard Operating Procedure for Kiln start up and operation. |  |  |  |  |  |  |  |  |  |

- The following parameters are to be maintained during normal operation

| S.NO | Operating Parameters | Permissible Range |  |
| --- | --- | --- | --- |
|  |  | KILN 1 | KILN 2 |
| 1 | KILN Feed Max | VP Pump Pressure 1200(mmWC) | 140 TPH |
| 2 | KILN Speed RPM | 4.2 | 5.2 |
| 3 | KILN Coal Feed rate(TPH) | 3.0 | 4.2 |
| 4 | P.C Coal Feed rate (TPH) | 4.5 | 8.5 |
| 5 | 5th cyclone material temp O C | 850 to 890 | 850 to 890 |
| 6 | Kiln Inlet temp O C | 950-1100 | 950-1150 |
| 7 | Clinker Liter weight (gm) | 1050-1250 | 1050-1250 |

Contingency Measures:

| Sr. No. | Contingency Measures | ## Reasons | Corrective Action |
| --- | --- | --- | --- |
| 1. | 5th Cyclone material temperature high. | - Excess PC coal. - Low kiln feed rate. | - Reduce PC coal. - Increase kiln feed rate. |
| 2. | 5th Cyclone material temperature low. | - Low PC coal quantity. - Excess kiln feed. | - Increase PC coal. - Reduce kiln feed. |
| 3. | Kiln inlet temperature high. | - Kiln coal quantity high. | - Reduce kiln coal quantity. |
| 4. | Kiln inlet temperature low. | - Kiln coal quantity low. | - Increase kiln coal quantity. |
| 5. | Clinker liter weight low. | - Low calcination. - Low Burning zone temperature. - Change in material characteristics. | - Increase 5th cyclone material temperature set point. - Increase kiln coal set point. - Inform QUAS department for corrective action. |
| 6. | Clinker liter weight high | - High calcination - High Burning Zone temperature - Change in material characteristics | - Reduce 5th material temperature set point - Reduce kiln coal set point - Inform QUAS department for corrective action |

7. CCR operator to be record the parameters hourly in APCL / PRO / F-03 or APCL / PRO / F-04

|  |  |  | ANJANI PORTLAND CEMENT LIMTIED |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Ref No. |  | OPER/SOP-2 |  | Dept: | Operations |  |  |
|  |  |  | Supersedes |  | Nil |  | Page : | 4 of 4 |  |  |
| Version No./Dt. |  | 01/01.03.2018 |  | Rev.No./Dt |  | 00/01.03.2018 | Next Review Dt. | 28.2.2021 | Effective Dt. | 01.03.2018 |
| Title | Standard Operating Procedure for Kiln start up and operation. |  |  |  |  |  |  |  |  |  |

8. The quality parameters are intimated by the shift chemist or Tester from the QUAS Dept

| # Particulate matter | # ≤ 30 mg/Nm3 |
| --- | --- |
| SOx | ≤ 100mg/Nm3 |
| NOx | ≤ 800mg/Nm3 |

- If any abnormal condition, inform Section head/HOD (Operation) and follow guidelines and instructions.

EnMS:

- Do not run idle running of the equipments

- Cross check the diesel flow rate is matching with diesel tank level indication to avoid excess flow rate

- Check the diesel or tyre oil stock is sufficient for light up to avoid failure of light up

- Do not waste /spillage  the diesel or tyre oil for torching for kiln light up

- Inform to CPP people to before start the preheater fan,RABH fan ,Kiln Main drive and Auxiliaries to get the power in time

- Calculate the power  loss across the  ID fans if any replaced with VFD

- Identify the interlocks and implement the same for energy saving opportunities